- Voltage Switch

- Latch

- D-Sub Connectors

- Ethernet Cable

- Power Cable

- D-Sub to CPC Cable

1

3

2

4

5

6

- Plug the power cable (4) into the wall.

- Connect the force box to the transducer (see table for appropriate voltage and connector).

- If using Ethernet, plug the cable (4) into the NI chassis and the computer. The force box can be opened with the latch (2). If using Wi-Fi, this will be covered later.

- Flip the voltage switch (1) to the appropriate voltage (same as the d-sub cable used).

- Turn on the computer and follow the force system instructions.

| Transducer | Exercise Device | Excitation Voltage | Connector |
| --- | --- | --- | --- |
|  | Force Chair Quad Exercise Device | 10V | D-Sub Note: This is the only transducer that connects directly to the force box |
|  | MR Quad Exercise Device | 10V | CPC |
|  | Hand Grip | 10V | CPC |
|  | Foot Exercise Device | 15V Note: Can operate with 10V | CPC |